[size=120][b]What is Ndless?[/b][/size]

Ndless is a program that allows your TI-Nspire calculator to run native applications, such as C, JavaScript, Python, and ARM assembly programs. In addition, Ndless will vastly expand your calculator's functionality, from PDF viewers to Gameboy Advance emulators. To see a complete list of Ndless Apps, go to [url]https://ndless-apps.org/[/url].

[size=120][b]Downloading Ndless[/b][/size]

[b]Before You Start[/b]

If installed properly, Ndless should not damage your calculator. However, given the potential risks, it's recommended that you back up your calculator's files.

Also, you will need to download the TI-Nspire Computer Link to transfer files to your calculator.

[b]Check Your OS[/b]

To check your operating system version, press [ON] - [5] - [4]. When you transfer the Ndless files to your calculator, make sure you send the files for your operating system!

[b]Preparation[/b]

1. Download [url=https://tiplanet.org/forum/archives_voir.php?id=44445]Ndless[/url] and extract the archived content.

2. On your calculator, create a top-level folder called “ndless”.

3. In this folder, transfer the ndless_resources.tns file and the ndless_installer_X.X.X.tns file for your OS from the extracted archive to your calculator using the TI-Nspire Computer Link.

[img]https://i.imgur.com/tDCdsZn.jpg[/img]

[b]Installation[/b]

Now open the ndless_installer_X.X.X.tns file. After about 30 seconds, a message should be displayed in the top left corner of the home screen. You have now successfully installed Ndless!

[img]https://i.imgur.com/6CL5Wv0.png[/img]

[i]Note: To uninstall Ndless, open the ndless_resources.tns file or reboot your calculator.[/i]

[i]Source: [url]http://ndless.me/[/url][/i]